Aircraft:	ANDREWKARMY RV-9A	(Van’s RV-9A)

Serial Number:	90374		  Registration Number:	N169AK

Aircraft Owner:	Andrew Karmy

Date of Inspection:			  Location of Inspection:	1S0

| Total Airframe Hours: |  |
| --- | --- |
| Total Engine Hours: |  |
| Total Propeller Hours: |  |

Inspection Performed By:				 Certificate:

### Preparation — Inspection Consumables

| Oil | 7 qts AeroShell 100WPlus |
| --- | --- |
| Oil filter | Champion CH48110 |
| Spark plugs | 8 - Unison UREM37BY |
| ELT batteries | 8 - Duracell MN1300 alkaline D-cell (every 2 years) |
| ELT remote battery | 1 - Duracell DL 1/3 NB lithium (every 8 years) |
| Air filter recharge kit | K&N 99-5000 |
| Brakes | Brake lining replacement kit |
| Tires/tubes – main | 2 - 5:00x5-6 ply, as needed |
| Tire/Tube – nose | 1 – 4:00x5 shinn, as needed |
| Wheel bearing grease | Hi-temp disc brake grease or Aeroshell 5 |
| Gasket lube | Dow DC-4 |
| Light lubricant | LPS-2 or comparable light lubricant for rod end bearings |

### Time-limited items

| Item | Last change | Next due |
| --- | --- | --- |
| ELT batteries (replace every 2 years) | Feb 2003 | Feb 2005 |
| ELT remote battery (replace every 8 years) | Feb 2003 | Feb 2011 |
| Altitude encoder & transponder certification | Feb 2003 | Feb 2005 |

### Empennage & Fuselage Group

- Remove empennage fairing, fuselage inspection & baggage wall covers

- Inspect vertical and horizontal stabilizer spar attach points

- Inspect elevator horns and attachments, and rear fuselage bulkhead interior

- Inspect all empennage structures for corrosion and condition

- Inspect all empennage attach bearings for condition/security, lubricate

- Inspect rudder control stops and cable attachments for security and cotter pin, rudder light wire for chafing

- Inspect elevator trim tab, arm, and actuator rod

- Inspect static ports, verify clear

- Inspect rear fuselage interior: check skin/bulkheads for cracking, damage, or corrosion, inspect all wiring & plumbing, clean as necessary

- Inspect midship elevator bellcrank assembly, lubricate rod end bearings

- Re-install rear baggage panels, rear fuse inspection plates, empennage fairing

### Landing Gear Group

- Remove nose wheel and nose wheel yoke, clean all parts, and assess wheel bearings, yoke bearings, and tire

- Clean & oil yoke bushing, re-install yoke and wheel replacing wheel and/or bearings as necessary

- Inspect nose wheel steering mechanism for breakout force and proper movement

- Re-install nose wheel, replace as necessary

- Remove wheel pants, upper & lower intersection fairings

- Inspect wheel pants and fairings for cracks, wear, loose or missing fasteners

- Jack plane one wheel at a time using jack adapter in wheel axle

- Remove brake assemblies and wheels one at a time

- Inspect tires for wear and cracks, rotate if indicated, replace if necessary

- Check brake cylinders for condition/leaks, brake lines for security and chafing, inspect brake linings for wear and condition, replace as necessary

- Re-pack wheel bearings (hi-temp disc brake grease) every other year

- Rotate or replace tires as necessary

- Clean and re-mount wheels and brake assemblies including cotter pin

- Check/set tire air pressure: 28-34 psi

- Clean wheel pants and re-install

- Inspect gear/fuselage attach bolts for security and tightness

- Inspect fuel vent ports for security and condition

- Re-install upper & lower intersection fairings, wheel pants

### Wing Group

- Remove wing inspection plates

- Inspect entire wing structure internally for cracks and corrosion including fuel tank attach bolts, special attention to main spar

- Inspect aileron control mechanism (push tubes, rod end bearings, bellcrank) for condition/security, lubricate rod end bearings

- Inspect wing root area including electrical connections and plumbing

- Inspect fuel cap and O ring condition

- Inspect aileron attach bolts/brackets for security, aileron control stops for security, lubricate rod end bearings

- Inspect pitot tube and plumbing for security and blockage

- Extend flaps, inspect flap actuator rods and attachment points for operation and security, lubricate rod end bearings

- Inspect strobe/nav lights for security and operation

- Inspect landing lights for security and operation

- Reinstall inspection plates and wing root fairings

### Cabin Group

- Remove flap motor cover, inspect flap motor mechanism & wiring for damage and proper operation, lubricate rod end bearing

- Remove seat pans for control inspection

- Inspect flight controls, lubricate rod end bearings

- Inspect condition of seat harnesses, mounts, and attachments

- Inspect gear tower area including spar bolts, antennas, wing wiring connections, leaks or chafing

- Inspect front-cabin console area including: fuel selector valve, fittings, fuel lines

- Check general condition under panel for loose wires, chafing, etc.

- Check all avionics and instruments for mounting security

- Check cabin heater and controls

- Check rudder pedals and brake cylinders for operation and leaks

- Remove and inspect under forward wiring tunnel for leaks and chafing

- Inspect cabin side of firewall area: aux fuel pump, firewall penetrations, wire bundles, fuel lines, vent tubes

- Inspect canopy structure and mechanism for security and wear

- Inspect canopy and windshield for cracks

- Check fire extinguisher condition

- Verify spare fuses available in rear seat pockets

- Re-install interior floor panels, seats, flap covers, and seat cushions

- Check/change ELT batteries, verify operation, set to “armed”

### Propeller Group

- Remove spinner noting orientation for replacement

- Inspect spinner and backplate for cracks or damage

- Inspect prop hub for cracks or leaks

- Inspect crankcase nose seal for oil leaks

- Inspect propeller blades for nicks, cracks and/or surface erosion

- Re-install spinner

### Engine Group

Service items:

- Remove engine cowl

- Clean cowling interior and inspect for cracks, distortion, loose or missing fasteners, heat damage

- Drain engine oil, collect oil sample for analysis, remove spin-on oil filter, replace and safety wire, refill engine oil per lubrication chart

- Cut open oil filter, inspect for particles

- Remove plugs noting current installed location

- Inspect spark plug condition, clean, re-gap to .016-.022”

- Perform leakdown compression test, note compression:

#1		 #2		 #3		 #4

- Check magneto timing & Electroair ignition condition and operation

- Reinstall spark plugs rotating top to bottom, apply anti-seize paste, torque to 400 in.lbs.

- Remove carburetor air box and carburetor heat attachments

- Service K&N air filter element every other year

- Reinstall air box and all attachments

- Remove and clean gascolator screen and bowl

- Remove and clean carburetor inlet strainer

- Inspect carburetor mounting nuts for security

- Check engine controls for proper movement and security

- Check exhaust system and mounting brackets for cracks

- Re-torque exhaust mounting nuts

Inspection items:

- Inspect accessories on rear engine case for security and leaks including: oil temp sender, oil pressure sender, magnetos, fuel pump, vacuum pump pad, oil lines & fittings, crankcase breather

- Inspect oil cooler condition including fins for damage or blocking, lines for leaks and/or chafing

- Inspect ignition harness, springs and insulators for condition

- Inspect starter: pinion & ring gears, mounts, wiring connections

- Inspect alternator: mounts, lead connections, belt condition and tension

- Inspect plenum/baffle for damage and air leaks

- Inspect engine mount system for damage and attachment security

- Inspect all firewall penetration seals

- Inspect fuel lines for condition, chafing, and signs of leaks

- Inspect all wire bundles for security and chafing

- Inspect all lines, hoses, and clamps for leaks, improper condition, looseness, and chafing

### Operational Inspection

- Verify all inspection panels and fairings secure

- Brake system check

Test run engine:

- Check oil pressure/temperature within limits

- Check aux fuel pump for proper fuel pressure

- Check ignition switch, both magnetos grounded check

- Check alternator shut off operation

- Check fuel gauges for operation

- Check static run up

- Magneto check

- Check for proper idle RPM

- Check RPM rise upon leaning mixture (idle mixture)

- Check fuel selector for engine shutoff

After shutdown:

- Inspect fwf area for fuel, oil, and hydraulic systems for leaks

- Re-install cowling

### Documentation

- Verify equipment list up to date

- Verify Registration, Airworthiness Certificate, and Operating Limitations are current and in aircraft

- Verify external data plate secure and installed

- Verify EXPERIMENTAL placard installed in cockpit

- Verify PASSENGER WARNING placard installed in cockpit

- Verify cockpit checklists and Pilot’s Operating Handbook in aircraft

- Review Airworthiness Directives for applicability and compliance

- Review Service Bulletins/Letters for compliance

- Check altitude encoder & transponder certification (every 24 months)

- Document Condition Inspection and record maintenance performed in airframe and engine logs

NOTES: