PREFLIGHT

DEPARTURE Checklists

- Control surfaces – SECURE

- Tires – INFLATED

- Fuel levels – CHECKED

- Fuel drains – SUMPED

- Engine oil – 5 QTS MIN.

- Pitot Cover– REMOVED

- Cowling – SECURE

- Right harness – FASTENED

- Loose items – SECURED

## START

- Switches – ALL OFF

- Avionics – ALL OFF

- Fuel selector – FULLEST

- Mixture - FULL RICH

- Master switch  – ON

- Flaps – RAISE?

- Mag & Ignition – ON

- Strobes – ON

- Carb heat – COLD

- Boost pump – ON

- Prime – AS REQUIRED

- Brakes – SET

- Throttle – CRACKED

- Propeller area – CLEAR

- START

- Oil pressure – > +25

- Avionics – ON

- Lights – AS NEEDED

## PRE-TAKEOFF

- Canopy – LATCHED

- Seat belts – FASTENED

- Brakes – SET

- Engine run-up – 1,800 rpm

- L /R magnetos – CHECK

- Carb heat – CHECK DROP

- Oil pres/temp – CHECK

- Fuel pressure – CHECK

- E-Bus Alt feed on

- Voltage – CHECK 14V

- E-Bus Alt feed off

- Altimeter - SET

- Avionics:

- XPNDR – ALT ON

- COM –FREQ SET

- GPS – WAYPOINT SET

- Flight controls – FREE

- Mixture – FULL RICH

(set for density altitude)

- Trim – SET TAKEOFF

- Flaps – AS NEEDED

- Boost pump – ON

- Strobes & Wig/Wag – ON

## CLIMBOUT

- Flaps – UP (before 85 mph)

- Boost pump – OFF

- Power reduction 1000 agl

- Maintain 110 mph +

Monitor CHTs

PRE-LANDING

ARRIVAL Checklists

- Seat belts – FASTENED

- Fuel selector – FULLEST TANK

- Mixture – FULL RICH

- Carb heat – ON (2200 rpm)

- Boost pump – ON

- Landing lights – Wig/Wag

LANDING SEQUENCE

- Pattern entry – 120 mph

Abeam numbers

- Power Idle, slow to 70 mph

- Extend flaps – HALF

- Power to 1000 RPM

- Trim for 70 mph

- Base – 70 mph

- Extend flaps – FULL

- Final – 70 mph

- Over fence – 65 mph

- Boost pump – OFF

- Flaps – RETRACT

- Trim – NEUTRAL

SHUTDOWN

- Flaps – EXTENDED?

- Flt Time – RECORD

- Avionics – OFF

- All switches – OFF

- Mixture – IDLE CUTOFF

- MAGS – OFF

EMERGENCY PROCEDURES

AVIATE - Establish best glide

NAVIGATE - Select landing area

COMMUNICATE – Contact ATC

AIRSPEEDS

- Best glide – 75 MPH

- Normal approach – 70 MPH

- Short/soft field – 60 MPH

ENGINE OUT

- CHECK:

- Mixture

- Carb heat

- Fuel selector

- Boost pump

- ATTEMPT RESTART

ENGINE OUT LANDING

- Fuel selector valve – OFF

- Seatbelts – TIGHT

- Squawk – 7700

- ELT – ACTIVATE

- Flaps – AS NEEDED

- Landing – SOFT or SHORT FIELD, 60 mph approach

RADIO FAILURE

- Squawk – 7600

- Green – CLEARED TO LAND

- Red – CIRCLE

- Flashing Red – UNSAFE

Speeds

Airspeeds & Weights

| VSO – Stall Speed – Landing Config | 45 MPH |
| --- | --- |
| VS – Stall Speed – Clean Config | 55 MPH |
| VX – Best Angle of Climb | 90 MPH |
| VY – Best Rate of Climb | 110 MPH |
| VFE – Max Flap Extension Speed | 85 MPH |
| VA – Design Maneuvering Speed | 118 MPH |
| VNO – Max Cruising Speed | 195 MPH |
| VNE – Never Exceed Speed | 210 MPH |
| Climb | 110 MPH |
| Cruise Climb | 140 MPH |
| Pattern | 120 MPH |
| Final Approach | 70 MPH |

## Weight & Balance

| Empty Weight | 1034 lbs |
| --- | --- |
| Gross Weight | 1750 lbs |
| Utility Category Gross Weight | 1600 lbs |
| Useful Load | 716 lbs |
| Max PAX Weight | 300 lbs |
| Max Baggage Weight | 100 lbs |

## Miscellaneous information

| Fuel Type | 100LL |
| --- | --- |
| Usable Fuel | 36 Gallons |
| Oil Capacity | 8 Quarts / 4 Quarts Min |
| Oil Type | Aero Shell 100W Plus |
| Electrical | 12 Volt / 40 AMP |
| Tire Pressure | Mains: 30 psi / Nose: 25 psi |

## Local Airport Information

| Thun Pierce County Unicom | 122.70 |
| --- | --- |
| Auburn Unicom | 122.80 |
| Flight Watch | 122.00 |
| FSS – Seattle – 800-992-7433 | 122.50 |
| Seattle Center |  |
| Seattle Departure – Thun | 126.50 |
| Seattle Departure – Auburn | 125.90 |

Battery Bus – 6 slots

Electrical BUS Fuse locations

|  | 7.5 – Electronic Ignition |
| --- | --- |
|  | 7.5 – Essential Bus Feed |
| 10 – Aux 12V outlet |  |

## Essential Bus – 10 slots

| 3 – Altitude Encoder | 5 – Fuel Boost Pump |
| --- | --- |
| 2 – Clock | 2 – FP5 Fuel Flow |
| 7.5 – SL40 Com | 1 – Co2 Alarm |
| 3 – SL70 Transponder | 3 – Engine Info Sys |
| 3 – SL50 GPS | 3 – Turn Coordinator |

## Main Bus – 20 slots

| 2 – PM3000 intercom | 10 – Left Landing Light |
| --- | --- |
| 2 – Low Volt warn | 10 – Right Landing Light |
| 5 – Strobes | 7.5 – Starter |
| 2 – SC2 Fuel Level | 10 – Flaps |
| 2 – Electric Elevator Trim | 1 – Panel Lights Dimmer |
|  | 10 – Nav Lights |
|  | 2 – Electric Aileron Trim |
|  | 2 – ANR Headset Power |
|  | 1 – Elevator Trim Indicator |